ASYC II multimeter

________________________________________________________________________________

Introduction:

This instrument driver provides programming support for ASYC II multimeter.

It contains functions for opening, configuring, taking measurements from, and

closing the instrument.

________________________________________________________________________________

Assumptions:

To successfully use this module, the following conditions must be met:

For GPIB instrument drivers:

-  the instrument is connected to the GPIB.

-  the GPIB address supplied to the initialize function must match the

GPIB address of the instrument.

For VXI instrument drivers:

-  the instrument is installed in the VXI mainframe and you are

using one of the following controller options:

Embedded controller

MXI

MXI2

GPIB-VXI

-  the logical address supplied to the initialize function must

match the logical address of the instrument.

For RS-232 instrument drivers:

-  the instrument is connected to the RS-232 interface.

-  the COM port, baud rate, parity, and timeout supplied to the

initialize function must match the settings of the instrument.

________________________________________________________________________________

Error and Status Information:

Error codes and warnings are returned as the return value of each instrument

driver function. A program should examine this value after each call to an

instrument driver function to determine if any error occurred.  The general

meaning of the return value is as follows:

Value                   Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

Possible error codes and their meanings are listed with the corresponding

instrument driver function.

________________________________________________________________________________

How To Use This Document:

This document is intended to be used as a programming reference manual.

It describes each function in the

ASYC II multimeter

instrument. The functions are listed in alphabetical order with a

description of the function, C syntax of the function, a description of each

parameter, and possible error codes.

________________________________________________________________________________

Function Tree Layout:

ASYC II multimeter                     Function Name:

Initialize                                  asyc2_init

Configure

Voltage                                  asyc2_voltage

Current                                  asyc2_current

dB                                       asyc2_dB

Test line                                asyc2_testline

Resistive Power                          asyc2_power

Frequency                                asyc2_frequency

Duty cycle                               asyc2_cycle

Pulse count                              asyc2_pulse_count

Pulse width                              asyc2_pulse_width

Resistance                               asyc2_resistance

Continuity test                          asyc2_continuity_test

Capacitance                              asyc2_capacitance

Diode voltage                            asyc2_diode

Temperature                              asyc2_temperature

Change mode

Resolution                               asyc2_resolution

Rejection                                asyc2_rejection

Input impedance                          asyc2_input_impedance

OHM reference                            asyc2_ref_ohm

Timer                                    asyc2_timer

LCD                                      asyc2_lcd

Measurement and control

Configuration                            asyc2_status

Measurement                              asyc2_data

Close                                       asyc2_close

________________________________________________________________________________

ASYC II multimeter

This instrument driver provides programming

support for ASYC2 multimeters.

Functions and Classes:

1. Initialize - Open the RS232 COM port,

initializes the instrument and stores the

configuration.

2. Configure - Configure the measurements for:

2.1.  Voltage

2.2.  Current

2.3.  dB

2.4.  Test line

2.5.  Resistive power

2.6.  Duty cycle

2.7.  Pulse Count

2.9.  Chronometer

2.10. Resistance

2.11. Continuity test

2.12. Capacitance

2.13. Diode Voltage

2.14. Temperature

3. Change mode - Configure the following modes:

3.1. Resolution

3.2. Rejection

3.3. Input Impedance

3.4. OHM reference

3.5. Timer

3.6. LCD

4. Measure - Take a measurement from the

instrument in the following mode:

4.1 MIN

4.2 MAX

4.3 AVG

4.4 Last

4.5 Next

5. Close - close the RS232 COM port.

________________________________________________________________________________

The following functions are in alphabetical order.

________________________________________________________________________________

asyc2_capacitance

int asyc2_capacitance (int range, int RELMode, double referenceF, int zoom,

int hold, int surv);

Purpose

Switches the instrument to Capacitance

measurement.

Parameter List

range

Variable Type       int

Specifies the range of the measurement.

Variable Type:  integer

Variable Range:  0 - AUTO Range

1 -   50 nF

2 -  500 nF

3 -    5 �F

4 -   50 �F

5 -  500 �F

6 - 5000 �F

7 -   50 mF

Default Value:   0 - AUTO Range

RELMode

Variable Type       int

Specifies the REL Mode. If it is

- Last Value:

the last value measured becomes the

reference value derived from subsequence

measurement

- Reference:

the reference value is the value

stored in the panel 'Reference'.

Variable Type:  integer

Variable Range:  2 - Reference

1 - Last Value

0 - Off

Default Value:   0 - Off

referenceF

Variable Type       double

Specifies the relative reference value.

Variable Type:   double

Variable Range:  according to the Range :

50mF    -50000    to +50000 �F

5000�F     -5000    to  +5000 �F

500�F      -500    to   +500 �F

50�F       -50    to    +50 �F

5�F        -5    to     +5 �F

500nF        -0.5  to      0.5 �F

50nF        -0.05 to      0.05�F

AUTO    -50000    to +50000 �F

Default Value:   0

zoom

Variable Type       int

Specifies the ZOOM mode to expand the bargraph

Variable Type:  integer

Variable Range:  0 - Off

1 - On

Default Value:   0 - Off

hold

Variable Type       int

Specifies the MEM mode to fix the display to the

current value.

Variable Type:  integer

Variable Range:  0 - Off

1 - On

Default Value:   0 - Off

surv

Variable Type       int

Specifies the SURV mode to access the surveillance

mode in which minimum, maximum and sliding average

values of the current measurement are stored.

Variable Type:  integer

Variable Range:  0 - Off

1 - On

Default Value:   0 - Off

Return Value

Displays the results of the instrument driver

function call:

Variable Type: Integer

Error Codes:

0   Success

-1   Range out of range

-2   REL Mode out of range

-3   Reference out of range

-4   Zoom out of range

-5   Memo out of range

-6   Surv out of range

220   Unable to open instrument

223   Instrument identification query failed

230   Error writing to instrument

231   Error reading from instrument

232   Instrument not initialized

240   Instrument timed out

300   Function not available

301   Wrong position of the rotary switch

302   Two arguments are incompatible

303   Measurement is overloaded (OL)

4xx   See RS232 Library error codes

(rs232err = 400 - Error)

________________________________________________________________________________

asyc2_close

int asyc2_close (void);

Purpose

Takes the instrument off-line.

Note:  The instrument must be reinitialized to use

it again.

Return Value

Displays the results of the instrument driver

function call:

Variable Type: Integer

Error Codes:

0   Success

221   Unable to close instrument

223   Instrument identification query failed

230   Error writing to instrument

231   Error reading from instrument

232   Instrument not initialized

240   Instrument timed out

300   Function not available

301   Wrong position of the rotary switch

302   Two arguments are incompatible

303   Measurement is overloaded (OL)

4xx   See RS232 Library error codes

(rs232err = 400 - Error)

________________________________________________________________________________

asyc2_continuity_test

int asyc2_continuity_test (int RELMode, double reference, int zoom, int hold,

int surv);

Purpose

Sets the instrument to Continuity Test

measurement.

Parameter List

RELMode

Variable Type       int

Specifies the REL Mode. If it is

- Last Value:

the last value measured becomes the

reference value derived from subsequence

measurement

- Reference:

the reference value is the value

stored in the panel 'Reference'.

Variable Type:  integer

Variable Range:  2 - Reference

1 - Last Value

0 - Off

Default Value:   0 - Off

reference

Variable Type       double

Specifies the relative reference value.

Variable Type:   double

Variable Range:  0 to +500 OHM

Default Value:   0

zoom

Variable Type       int

Specifies the ZOOM mode to expand the bargraph

Variable Type:  integer

Variable Range:  0 - Off

1 - On

Default Value:   0 - Off

hold

Variable Type       int

Specifies the MEM mode to fix the display to the

current value.

Variable Type:  integer

Variable Range:  0 - Off

1 - On

Default Value:   0 - Off

surv

Variable Type       int

Specifies the SURV mode to access the surveillance

mode in which minimum, maximum and sliding average

values of the current measurement are stored.

Variable Type:  integer

Variable Range:  0 - Off

1 - On

Default Value:   0 - Off

Return Value

Displays the results of the instrument driver

function call:

Variable Type: Integer

Error Codes:

0   Success

-1   REL Mode out of range

-2   Reference out of range

-3   Zoom out of range

-4   Memo out of range

-5   Surv out of range

220   Unable to open instrument

223   Instrument identification query failed

230   Error writing to instrument

231   Error reading from instrument

232   Instrument not initialized

240   Instrument timed out

300   Function not available

301   Wrong position of the rotary switch

302   Two arguments are incompatible

303   Measurement is overloaded (OL)

4xx   See RS232 Library error codes

(rs232err = 400 - Error)

________________________________________________________________________________

asyc2_current

int asyc2_current (int function, int range, int RELMode, double referencemA,

int zoom, int hold, int surv);

Purpose

Switches the instrument to current measurement.

Parameter List

function

Variable Type       int

Specifies the type of measurement.

Variable Type: integer

Valid Range:   0 -  DC

1 -  AC

2 -  AC+DC

3 -  Peak+

4 -  Peak-

Default Value: 0 -  DC

range

Variable Type       int

Specifies the range of the measurement

(auto ranging is disable with Peak measurement).

Variable Type:  integer

Variable Range:  0 - AUTO

1 - 500 �A

2 - 5 mA

3 - 50 mA

4 - 500 mA

5 - 10 A

Default Value:   0 - AUTO

RELMode

Variable Type       int

Specifies the REL Mode. If it is

- Last Value:

the last value measured becomes the

reference value derived from subsequence

measurement

- Reference:

the reference value is the value

stored in the panel 'Reference'.

Variable Type:  integer

Variable Range:  2 - Reference

1 - Last Value

0 - Off

Default Value:   0 - Off

referencemA

Variable Type       double

Specifies the relative reference value.

Variable Type:   double

Variable Range: according to the range :

10 A    -10000 to +10000mA

500mA      -500 to   +500mA

50mA       -50 to    +50mA

5mA        -5 to     +5mA

500�A      -0.5 to   +0.5mA

AUTO      -500 to   +500mA

Default Value:  0

zoom

Variable Type       int

Specifies the ZOOM mode to expand the bargraph.

Variable Type:  integer

Variable Range:  0 - Off

1 - On

Default Value:   0 - Off

hold

Variable Type       int

Specifies the MEM mode to fix the display to the

current value.

Variable Type:  integer

Variable Range:  0 - Off

1 - On

Default Value:   0 - Off

surv

Variable Type       int

Specifies the SURV mode to access the surveillance

mode in which minimum, maximum and sliding average

values of the current measurement are stored.

Variable Type:  integer

Variable Range:  0 - Off

1 - On

Default Value:   0 - On

Return Value

Displays the results of the instrument driver

function call:

Variable Type: Integer

Error Codes:

0   Success

-1   Config out of range

-2   Range out of range

-3   REL Mode of range

-4   Reference out of range

-5   Zoom out of range

-6   Memo out of range

-7   Surv out of range

220   Unable to open instrument

223   Instrument identification query failed

230   Error writing to instrument

231   Error reading from instrument

232   Instrument not initialized

240   Instrument timed out

300   Function not available

301   Wrong position of the rotary switch

302   Two arguments are incompatible

303   Measurement is overloaded (OL)

4xx   See RS232 Library error codes

(rs232err = 400 - Error)

________________________________________________________________________________

asyc2_cycle

int asyc2_cycle (int sign, int RELMode, double reference, int hold, int surv);

Purpose

Switches the instrument to Duty Cycle measurement.

Parameter List

sign

Variable Type       int

Specifies the duty cycle sign:

+ : positive part of the signal

- : negative part of the signal

Variable Type:  integer

Variable Range:  0 - positive part

1 - negative part

Default Value:   0

RELMode

Variable Type       int

Specifies the REL Mode. If it is

- Last Value:

the last value measured becomes the

reference value derived from subsequence

measurement

- Reference:

the reference value is the value

stored in the panel 'Reference'.

Variable Type:  integer

Variable Range:  2 - Reference

1 - Last Value

0 - Off

Default Value:   0 - Off

reference

Variable Type       double

Specifies the relative reference value.

Variable Type:   double

Variable Range: 0 to 100%

Default Value:   0

hold

Variable Type       int

Specifies the MEM mode to fix the display to the

current value.

Variable Type:  integer

Variable Range:  0 - Off

1 - On

Default Value:   0 - Off

surv

Variable Type       int

Specifies the SURV mode to access the surveillance

mode in which minimum, maximum and sliding average

values of the current measurement are stored.

Variable Type:  integer

Variable Range:  0 - Off

1 - On

Default Value:   0 - Off

Return Value

Displays the results of the instrument driver

function call:

Variable Type: Integer

Error Codes:

0   Success

-1   Sign out of range

-2   REL Mode out of range

-3   Reference out of range

-4   Memo out of range

-5   Surv out of range

220   Unable to open instrument

223   Instrument identification query failed

230   Error writing to instrument

231   Error reading from instrument

232   Instrument not initialized

240   Instrument timed out

300   Function not available

301   Wrong position of the rotary switch

302   Two arguments are incompatible

303   Measurement is overloaded (OL)

4xx   See RS232 Library error codes

(rs232err = 400 - Error)

________________________________________________________________________________

asyc2_data

int asyc2_data (int measure, double *result, int *type);

Purpose

Define the measurements to execute :

MIN, MAX,AVG, Last, Next.

MIN : minimum value stored in SURVeillance mode.

MAX : maximum value stored in SURVeillance mode.

AVG : sliding value stored in SURVeillance mode.

Last: last value measured.

Next: next value measured.

Parameter List

measure

Variable Type       int

Specifies the type of the measurement.

Variable Type:  integer

Variable Range:  0 - MIN

1 - MAX

2 - AVG

3 - last

4 - next

Default Value:   0 - MIN

Notes :

MIN : minimum value stored in SURVeillance mode.

MAX : maximum value stored in SURVeillance mode.

AVG : sliding value stored in SURVeillance mode.

Last: last value measured.

Next: next value measured.

result

Variable Type       double (passed by reference)

Displays the results of the measurement in

Scientific Format. If you want to display the

results in Floating Point Format, Use the menu

Options/Edit Panel then double click on the

Measurement control and set the Display Format

Floating Point. Use the menu Options/Operate Panel

to take control.

The unities are :

VOLT        for voltage

AMPERE      for current

OHM         for resistance

Hz          for frequency

%           for duty cycle

F           for capacity

�C or �F    for temperature

Variable Type:  Double

Note: If Error 300 is displayed (Not available),

this result is not valid.

type

Variable Type       int (passed by reference)

Displays the configuration of the measurement.

0 => A DC      14 => OHM      26 => Power

2 => A Peak+   15 => Capa     27 =>

3 => A Peak-   16 => V DC     41 => Frequency

4 => A AC      18 => V Peak+  42 => P Width+

6 => A AC+DC   19 => V Peak-  43 => P Width-

10 => Temp      20 => V AC     44 => D.Cycl.+

11 => Continu   22 => V AC+DC  45 => D.Cycl.-

12 =>           24 => dB       46 => Cmpt+

13 => Diode     25 => Test L.  47 => Cmpt-

Variable Type: integer

Note: If Error 300 is displayed (Not available),

this result is not valid.

Return Value

Displays the results of the instrument driver

function call:

Variable Type: Integer

Error Codes:

0   Success

-1   Type of measurement out of range

220   Unable to open instrument

223   Instrument identification query failed

230   Error writing to instrument

231   Error reading from instrument

232   Instrument not initialized

240   Instrument timed out

300   Function not available

301   Wrong position of the rotary switch

302   Two arguments are incompatible

303   Measurement is overloaded (OL)

4xx   See RS232 Library error codes

(rs232err = 400 - Error)

________________________________________________________________________________

asyc2_dB

int asyc2_dB (int RELMode, double reference, int surv, int hold);

Purpose

Sets the instrument to dBm measurement.

This function is available on VAC.

Parameter List

RELMode

Variable Type       int

Specifies the REL Mode. If it is

- Last Value:

the last value measured becomes the

reference value derived from subsequence

measurement

- Reference:

the reference value is the value

stored in the panel 'Reference'.

Variable Type:  integer

Variable Range:  2 - Reference

1 - Last Value

0 - Off

Default Value:   0 - Off

reference

Variable Type       double

Specifies the relative reference value.

Variable Type:   double

Variable Range: -500.0 to +500.0 dB

Default Value:   0

surv

Variable Type       int

Specifies the SURV mode to access the surveillance

mode in which minimum, maximum and sliding average

values of the current measurement are stored.

Variable Type:  integer

Variable Range:  0 - Off

1 - On

Default Value:   0 - Off

hold

Variable Type       int

Specifies the MEM mode to fix the display to the

current value.

Variable Type:  integer

Variable Range:  0 - Off

1 - On

Default Value:   0 - Off

Return Value

Displays the results of the instrument driver

function call:

Variable Type: Integer

Error Codes:

0   Success

-1   REL Mode out of range

-2   Reference out of range

-3   Surv out of range

-4   Memo out of range

220   Unable to open instrument

223   Instrument identification query failed

230   Error writing to instrument

231   Error reading from instrument

232   Instrument not initialized

240   Instrument timed out

300   Function not available

301   Wrong position of the rotary switch

302   Two arguments are incompatible

303   Measurement is overloaded (OL)

4xx   See RS232 Library error codes

(rs232err = 400 - Error)

________________________________________________________________________________

asyc2_diode

int asyc2_diode (int RELMode, double reference, int hold, int surv);

Purpose

Switches the instrument to Diode voltage

measurement.

Check if the rotary switch is on Diode (or -II-)

position.

Parameter List

RELMode

Variable Type       int

Specifies the REL Mode. If it is

- Last Value:

the last value measured becomes the

reference value derived from subsequence

measurement

- Reference:

the reference value is the value

stored in the panel 'Reference'.

Variable Type:  integer

Variable Range:  2 - Reference

1 - Last Value

0 - Off

Default Value:   0 - Off

reference

Variable Type       double

Specifies the relative reference value.

Variable Type:   double

Variable Range:  0 to +2 VOLT

Default Value:   0

hold

Variable Type       int

Specifies the MEM mode to fix the display to the

current value.

Variable Type:  integer

Variable Range:  0 - Off

1 - On

Default Value:   0 - Off

surv

Variable Type       int

Specifies the SURV mode to access the surveillance

mode in which minimum, maximum and sliding average

values of the current measurement are stored.

Variable Type:  integer

Variable Range:  0 - Off

1 - On

Default Value:   0 - Off

Return Value

Displays the results of the instrument driver

function call:

Variable Type: Integer

Error Codes:

0   Success

-1   REL Mode out of range

-2   Reference out of range

-3   Memo out of range

-4   Surv out of range

220   Unable to open instrument

223   Instrument identification query failed

230   Error writing to instrument

231   Error reading from instrument

232   Instrument not initialized

240   Instrument timed out

300   Function not available

301   Wrong position of the rotary switch

302   Two arguments are incompatible

303   Measurement is overloaded (OL)

4xx   See RS232 Library error codes

(rs232err = 400 - Error)

________________________________________________________________________________

asyc2_frequency

int asyc2_frequency (int RELMode, int zoom, int hold, int surv);

Purpose

Sets the instrument to frequency measurement.

Parameter List

RELMode

Variable Type       int

Specifies the REL mode. If it is

- On: the last value measured becomes the

reference value derived from subsequence

measurement

Variable Type:  integer

Variable Range:  0 - Off

1 - On

Default Value:   0

zoom

Variable Type       int

Specifies the ZOOM mode to expand the bargraph

Variable Type:  integer

Variable Range:  0 - Off

1 - On

Default Value:   0 - Off

hold

Variable Type       int

Specifies the MEM mode to fix the display to the

current value.

Variable Type:  integer

Variable Range:  0 - Off

1 - On

Default Value:   0 - Off

surv

Variable Type       int

Specifies the SURV mode to access the surveillance

mode in which minimum, maximum and sliding average

values of the current measurement are stored.

Variable Type:  integer

Variable Range:  0 - Off

1 - On

Default Value:   0 - Off

Return Value

Displays the results of the instrument driver

function call:

Variable Type: Integer

Error Codes:

0   Success

-1   REL Mode out of range

-2   Zoom out of range

-3   Memo out of range

-4   Surv out of range

220   Unable to open instrument

223   Instrument identification query failed

230   Error writing to instrument

231   Error reading from instrument

232   Instrument not initialized

240   Instrument timed out

300   Function not available

301   Wrong position of the rotary switch

302   Two arguments are incompatible

303   Measurement is overloaded (OL)

4xx   See RS232 Library error codes

(rs232err = 400 - Error)

________________________________________________________________________________

asyc2_init

int asyc2_init (int COMPort);

Purpose

Initializes the instrument module by opening a

COM port, and reads the status of the DMM ASYC II.

Parameter List

COMPort

Variable Type       int

Specifies the COM port for the RS232.

Variable Type: integer

Valid Range:   1 - COM1

2 - COM2

Default Value: 1 - COM1

Return Value

Displays the results of the instrument driver

function call:

Variable Type: integer

Error Codes:

0   Success

-1   COM port out of range

220   Unable to open instrument

223   Instrument identification query failed

230   Error writing to instrument

231   Error reading from instrument

232   Instrument not initialized

239   Error in configuring timeout

240   Instrument timed out

4xx   See RS232 Library error codes

(rs232err = 400 - Error)

________________________________________________________________________________

asyc2_input_impedance

int asyc2_input_impedance (int inputimpedance);

Purpose

Changes the Input impedance of the instrument. It

is used for measurement in the mV range.

Parameter List

inputimpedance

Variable Type       int

Specifies the input impedance for measurement in

the mV range

Variable Type:  integer

Variable Range:  0 -     10 MOHM

1 - >10000 MOHM

Default Value:   0 -     10 MOHM

Return Value

Displays the results of the instrument driver

function call:

Variable Type: Integer

Error Codes:

0   Success

-1   Input impedance out of range

220   Unable to open instrument

223   Instrument identification query failed

230   Error writing to instrument

231   Error reading from instrument

232   Instrument not initialized

240   Instrument timed out

4xx   See RS232 Library error codes

(rs232err = 400 - Error)

________________________________________________________________________________

asyc2_lcd

int asyc2_lcd (int status);

Purpose

Changes the status of the LCD. It can display a

measurement or all the segments.

Parameter List

status

Variable Type       int

Specifies the status of the display:

- Normal display: displays a measurement

- Full display  : all segments are displayed

Variable Type:  integer

Variable Range:  0 - Normal display

1 - full display

Default Value:   0 - Normal display

Return Value

Displays the results of the instrument driver

function call:

Variable Type: Integer

Error Codes:

0   Success

-1   Status out of range

220   Unable to open instrument

223   Instrument identification query failed

230   Error writing to instrument

231   Error reading from instrument

232   Instrument not initialized

240   Instrument timed out

4xx   See RS232 Library error codes

(rs232err = 400 - Error)

________________________________________________________________________________

asyc2_power

int asyc2_power (int RELMode, int hold, int surv);

Purpose

Sets the instrument to resistive Power

measurement.

This function is available on VDC.

Parameter List

RELMode

Variable Type       int

Specifies the REL mode. If it is

- On: the last value measured becomes the

reference value derived from subsequence

measurement

Variable Type:  integer

Variable Range:  0 - Off

1 - On

Default Value:   0

hold

Variable Type       int

Specifies the MEM mode to fix the display to the

current value.

Variable Type:  integer

Variable Range:  0 - Off

1 - On

Default Value:   0 - Off

surv

Variable Type       int

Specifies the SURV mode to access the surveillance

mode in which minimum, maximum and sliding average

values of the current measurement are stored.

Variable Type:  integer

Variable Range:  0 - Off

1 - On

Default Value:   0 - Off

Return Value

Displays the results of the instrument driver

function call:

Variable Type: Integer

Error Codes:

0   Success

-1   REL Mode out of range

-2   Memo out of range

-3   Surv out of range

220   Unable to open instrument

223   Instrument identification query failed

230   Error writing to instrument

231   Error reading from instrument

232   Instrument not initialized

240   Instrument timed out

300   Function not available

301   Wrong position of the rotary switch

302   Two arguments are incompatible

303   Measurement is overloaded (OL)

4xx   See RS232 Library error codes

(rs232err = 400 - Error)

________________________________________________________________________________

asyc2_pulse_count

int asyc2_pulse_count (int sign);

Purpose

Switches the instrument to Pulse Count

measurement.

Parameter List

sign

Variable Type       int

Specifies the pusle count sign:

+ : count the positive pulses

- : count the negative pulses

Variable Type:  integer

Variable Range:  0 - positive pulses

1 - negative pulses

Default Value:   0

Return Value

Displays the results of the instrument driver

function call:

Variable Type: Integer

Error Codes:

0   Success

-1   Sign out of range

220   Unable to open instrument

223   Instrument identification query failed

230   Error writing to instrument

231   Error reading from instrument

232   Instrument not initialized

240   Instrument timed out

300   Function not available

301   Wrong position of the rotary switch

302   Two arguments are incompatible

303   Measurement is overloaded (OL)

4xx   See RS232 Library error codes

(rs232err = 400 - Error)

________________________________________________________________________________

asyc2_pulse_width

int asyc2_pulse_width (int sign, int hold, int surv);

Purpose

Switches the instrument to positive or negative

pulse width measurement.

Parameter List

sign

Variable Type       int

Specifies the chrono sign:

+ : count the positive pulses

- : count the negative pulses

Variable Type:  integer

Variable Range:  0 - positive pulses

1 - negative pulses

Default Value:   0

hold

Variable Type       int

Specifies the MEM mode to fix the display to the

current value.

Variable Type:  integer

Variable Range:  0 - Off

1 - On

Default Value:   0 - Off

surv

Variable Type       int

Specifies the SURV mode to access the surveillance

mode in which minimum, maximum and sliding average

values of the current measurement are stored.

Variable Type:  integer

Variable Range:  0 - Off

1 - On

Default Value:   0 - Off

Return Value

Displays the results of the instrument driver

function call:

Variable Type: Integer

Error Codes:

0   Success

-1   Sign out of range

-2   Memo out of range

-3   Surv out of range

220   Unable to open instrument

223   Instrument identification query failed

230   Error writing to instrument

231   Error reading from instrument

232   Instrument not initialized

240   Instrument timed out

300   Function not available

301   Wrong position of the rotary switch

302   Two arguments are incompatible

303   Measurement is overloaded (OL)

4xx   See RS232 Library error codes

(rs232err = 400 - Error)

________________________________________________________________________________

asyc2_ref_ohm

int asyc2_ref_ohm (int value);

Purpose

Change the OHM value reference of the instrument.

It is used for dB or resistive power measurement.

Parameter List

value

Variable Type       int

Specifies the OHM value reference in dB and

resistive power measurement.

Variable Type:  integer

Variable Range:  1 to 9999 (OHM)

Default Value:   600

Return Value

Displays the results of the instrument driver

function call:

Variable Type: Integer

Error Codes:

0   Success

-1   OHM value reference out of range

220   Unable to open instrument

223   Instrument identification query failed

230   Error writing to instrument

231   Error reading from instrument

232   Instrument not initialized

240   Instrument timed out

4xx   See RS232 Library error codes

(rs232err = 400 - Error)

________________________________________________________________________________

asyc2_rejection

int asyc2_rejection (int rejection);

Purpose

Changes the Rejection of the instrument (50 or 60

Hz).

Parameter List

rejection

Variable Type       int

Specifies the rejection of the instrument

Variable Type:  integer

Variable Range:  0 - 50 Hz

1 - 60 Hz

Default Value:   0 - 50 Hz

Return Value

Displays the results of the instrument driver

function call:

Variable Type: Integer

Error Codes:

0   Success

-1   Rejection out of range

220   Unable to open instrument

223   Instrument identification query failed

230   Error writing to instrument

231   Error reading from instrument

232   Instrument not initialized

240   Instrument timed out

4xx   See RS232 Library error codes

(rs232err = 400 - Error)

________________________________________________________________________________

asyc2_resistance

int asyc2_resistance (int range, int RELMode, double referencekOHM, int zoom,

int hold, int surv);

Purpose

Sets the instrument to Resistance measurement.

(Check that the rotary switch is in OHM position).

Parameter List

range

Variable Type       int

Specifies the range of the measurement

(auto ranging is disable with Peak measurement).

Variable Type:  integer

Variable Range:  0 - AUTO

1 - 500  OHM

2 -   5 kOHM

3 -  50 kOHM

4 - 500 kOHM

5 -   5 MOHM

6 -  50 MOHM

Default Value:   0 - AUTO

RELMode

Variable Type       int

Specifies the REL Mode. If it is

- Last Value:

the last value measured becomes the

reference value derived from subsequence

measurement

- Reference:

the reference value is the value

stored in the panel 'Reference'.

Variable Type:  integer

Variable Range:  2 - Reference

1 - Last Value

0 - Off

Default Value:   0 - Off

referencekOHM

Variable Type       double

Specifies the relative reference value.

Variable Type:   double

Variable Range:  according to the Range :

50MOHM       -50000 to   +50000 kOHM

5MOHM        -5000 to    +5000 kOHM

500kOHM         -500 to     +500 kOHM

50kOHM          -50 to      +50 kOHM

5kOHM           -5 to       +5 kOHM

500 OHM           -0.5 to     +0.5 kOHM

AUTO       -50000   to +50000 kOHM

Default Value:   0

zoom

Variable Type       int

Specifies the ZOOM mode to expand the bargraph

Variable Type:  integer

Variable Range:  0 - Off

1 - On

Default Value:   0 - Off

hold

Variable Type       int

Specifies the MEM mode to fix the display to the

current value.

Variable Type:  integer

Variable Range:  0 - Off

1 - On

Default Value:   0 - Off

surv

Variable Type       int

Specifies the SURV mode to access the surveillance

mode in which minimum, maximum and sliding average

values of the current measurement are stored.

Variable Type:  integer

Variable Range:  0 - Off

1 - On

Default Value:   0 - Off

Return Value

Displays the results of the instrument driver

function call:

Variable Type: Integer

Error Codes:

0   Success

-1   Range out of range

-2   REL Mode out of range

-3   Reference out of range

-4   Zoom out of range

-5   Memo out of range

-6   Surv out of range

220   Unable to open instrument

223   Instrument identification query failed

230   Error writing to instrument

231   Error reading from instrument

232   Instrument not initialized

240   Instrument timed out

300   Function not available

301   Wrong position of the rotary switch

302   Two arguments are incompatible

303   Measurement is overloaded (OL)

4xx   See RS232 Library error codes

(rs232err = 400 - Error)

________________________________________________________________________________

asyc2_resolution

int asyc2_resolution (int resolution);

Purpose

Changes the resolution of the instrument (5.000 or

50.000 points).

Parameter List

resolution

Variable Type       int

Specifies the resolution of the display

- Low   :  5.000 points

- High  : 50.000 points

Variable Type:  integer

Variable Range:  0 -  5.000 points

1 - 50.000 points

Default Value:   1

Return Value

Displays the results of the instrument driver

function call:

Variable Type: Integer

Error Codes:

0   Success

-1   Resolution out of range

220   Unable to open instrument

223   Instrument identification query failed

230   Error writing to instrument

231   Error reading from instrument

232   Instrument not initialized

240   Instrument timed out

302   Resolution not compatible with

measurement mode. See Notes below.

4xx   See RS232 Library error codes

(rs232err = 400 - Error)

Notes :

High resolution not compatible with:

Test Line

Duty Cycle +/-

Capacitance

Diode

Peak +/- (A,V)

Low resolution not compatible with:

dB

Frequency

Count +/-

P Width +/-

Power

Temp

________________________________________________________________________________

asyc2_status

int asyc2_status (int *instrument, int *version, long *calibrationDate,

int *resolution, int *rejection, int *rotaryswitch,

int *measure, int *range, int *inputZi, int *ohmReference,

long *timer, int *temperature, int *fuses, int *battery);

Purpose

Configuration: return data informations from the

instrument.

Parameter List

instrument

Variable Type       int (passed by reference)

Displays the reference of the instrument:

0, 1, 2, 3.

Variable Type : integer

version

Variable Type       int (passed by reference)

Displays the version of the instrument.

0 :  First Version

1 :  Second Version

Variable Type: integer

calibrationDate

Variable Type       long (passed by reference)

Displays the date of the last calibration

(dd/mm/yy).

Variable Type: long

Example  : 51095

day    :  5

month  : 10

year   : 95

resolution

Variable Type       int (passed by reference)

Displays the resolution of the instrument.

0 => Low   (5.000 points)

1 => High (50.000 points)

Variable Type: integer

rejection

Variable Type       int (passed by reference)

Displays the rejection of the instrument.

0 => 50 Hz

1 => 60 Hz

Variable Type: integer

rotaryswitch

Variable Type       int (passed by reference)

Displays the position of the rotary switch.

0 => OHM             8 => mA

1 => CAPA            9 => 10A

2 => Not Used       10 => 5mA

3 => TEMP           11 => 50mA

4 => VAC            12 => 500mA

5 => mV

6 => VDC

7 => FUSE2

Variable Type: integer

measure

Variable Type       int (passed by reference)

Displays the configuration of the measurement.

0 => A DC      14 => OHM      26 => Power

2 => A Peak+   15 => Capa     27 =>

3 => A Peak-   16 => V DC     41 => Frequency

4 => A AC      18 => V Peak+  42 => P Width+

6 => A AC+DC   19 => V Peak-  43 => P Width-

10 => Temp      20 => V AC     44 => D.Cycl.+

11 => Continu   22 => V AC+DC  45 => D.Cycl.-

12 =>           24 => dB       46 => Count+

13 => Diode     25 => Test L.  47 => Count-

Variable Type: integer

range

Variable Type       int (passed by reference)

Displays the range of the measurement.

Volt      Ampere       Ohm        Capa

0 =>  500mV     500�A     500 Ohm      50nF

1 =>    5 V       5mA       5kOhm     500nF

2 =>   50 V      50mA      50kOhm       5�F

3 =>  500 V     500mA     500kOhm      50�F

4 => 1000 V                 5MOhm     500�F

5 =>             10 A      50MOhm    5000�F

6 =>                                   50mF

Variable Type: integer

inputZi

Variable Type       int (passed by reference)

Displays the input impedance of the instrument.

0 =>       10 MOhms

1 =>   > 1000 MOhms

Variable Type: integer

ohmReference

Variable Type       int (passed by reference)

Displays the ohm reference value for dBm and

resistance power measurement.

Variable Type: integer

timer

Variable Type       long (passed by reference)

Displays the timer value for repetitive

measurement (ex: 31542 ==> 3h 15' 42").

Variable Type: long

temperature

Variable Type       int (passed by reference)

Displays the temperature unity for temperature

measurement.

0 => �Celsius

1 => �Fahrenheit.

Variable Type: integer

fuses

Variable Type       int (passed by reference)

Displays the status of the two fuses :

Fuse 2     Fuse 1

0 =>  OFF        OFF

1 =>  OFF         ON

10 =>   ON        OFF

11 =>   ON         ON

(OFF means that the fuse is blown)

Variable Type: integer

Variable Name: asyc2_fuse1

battery

Variable Type       int (passed by reference)

Displays the status of the battery

0 => battery ok

1 => battery out

Variable Type: integer

Return Value

Displays the results of the instrument driver

function call:

Variable Type: Integer

Error Codes:

0   Success

220   Unable to open instrument

223   Instrument identification query failed

230   Error writing to instrument

231   Error reading from instrument

232   Instrument not initialized

240   Instrument timed out

4xx   See RS232 Library error codes

(rs232err = 400 - Error)

________________________________________________________________________________

asyc2_temperature

int asyc2_temperature (int unit, int sensor, int hold, int surv, int RELMode,

double reference);

Purpose

Sets the instrument to Temperature measurement.

Parameter List

unit

Variable Type       int

Specifies the unit of the temperature.

- �Celsius

- �Fahrenheit.

Variable Type:  integer

Variable Range:  0 - �Celsius

1 - �Fahrenheit

Default Value:   0 - �Celsius

sensor

Variable Type       int

Specifies the sensor selection.

Variable Type:  integer

Variable Range:  0 - Pt 1000

1 - Pt  100

Default Value:   1 - Pt  100

hold

Variable Type       int

Specifies the MEM mode to fix the display to the

current value.

Variable Type:  integer

Variable Range:  0 - Off

1 - On

Default Value:   0 - Off

surv

Variable Type       int

Specifies the SURV mode to access the surveillance

mode in which minimum, maximum and sliding average

values of the current measurement are stored.

Variable Type:  integer

Variable Range:  0 - Off

1 - On

Default Value:   0 - Off

RELMode

Variable Type       int

Specifies the REL Mode. If it is

- Last Value:

the last value measured becomes the

reference value derived from subsequence

measurement

- Reference:

the reference value is the value

stored in the panel 'Reference'.

Variable Type:  integer

Variable Range:  2 - Reference

1 - Last Value

0 - Off

Default Value:   0 - Off

reference

Variable Type       double

Specifies the relative reference value.

Variable Type:   double

Variable Range: PT100  : 0 to +500 OHM

PT1000 : 0 to +5.000 OHM

Default Value:   0

Return Value

Displays the results of the instrument driver

function call:

Variable Type: Integer

Error Codes:

0   Success

-1   Unit out of range

-2   Sensor out of range

-3   Memo out of range

-4   Surv out of range

-5   REL Mode out of range

-6   Reference out of range

220   Unable to open instrument

223   Instrument identification query failed

230   Error writing to instrument

231   Error reading from instrument

232   Instrument not initialized

240   Instrument timed out

300   Function not available

301   Wrong position of the rotary switch

302   Two arguments are incompatible

303   Measurement is overloaded (OL)

4xx   See RS232 Library error codes

(rs232err = 400 - Error)

________________________________________________________________________________

asyc2_testline

int asyc2_testline (int RELMode, double reference, int hold, int surv);

Purpose

Switches the instrument to Testline measurement.

This function is available on VAC.

Parameter List

RELMode

Variable Type       int

Specifies the REL Mode. If it is

- Last Value:

the last value measured becomes the

reference value derived from subsequence

measurement

- Reference:

the reference value is the value

stored in the panel 'Reference'.

Variable Type:  integer

Variable Range:  2 - Reference

1 - Last Value

0 - Off

Default Value:   0 - Off

reference

Variable Type       double

Specifies the relative reference value.

Variable Type:   double

Variable Range: 0 to +600 Volts

Default Value:   0

hold

Variable Type       int

Specifies the MEM mode to fix the display to the

current value.

Variable Type:  integer

Variable Range:  0 - Off

1 - On

Default Value:   0 - Off

surv

Variable Type       int

Specifies the SURV mode to access the surveillance

mode in which minimum, maximum and sliding average

values of the current measurement are stored.

Variable Type:  integer

Variable Range:  0 - Off

1 - On

Default Value:   0 - Off

Return Value

Displays the results of the instrument driver

function call:

Variable Type: Integer

Error Codes:

0   Success

-1   REL Mode out of range

-2   Rel. reference out of range

-3   Memo out of range

-4   Surv out of range

220   Unable to open instrument

223   Instrument identification query failed

230   Error writing to instrument

231   Error reading from instrument

232   Instrument not initialized

240   Instrument timed out

300   Function not available

301   Wrong position of the rotary switch

302   Two arguments are incompatible

303   Measurement is overloaded (OL)

4xx   See RS232 Library error codes

(rs232err = 400 - Error)

________________________________________________________________________________

asyc2_timer

int asyc2_timer (char timervalue[]);

Purpose

Changes the Timer value of the instrument. It is

used for repetitive measurement.

Parameter List

timervalue

Variable Type       char []

Specifies the timer value for repetitive

measurement:

1st character: hour   (0 to 9)

2nd character: minute (0 to 5)

3rd character: minute (0 to 9)

4th character: second (0 to 5)

5th character: second (0 to 9)

Variable Type:  string

Variable Range:  "0" to "95959"

Default Value:   "1"

Return Value

Displays the results of the instrument driver

function call:

Variable Type: Integer

Error Codes:

0   Success

-1   Timer value out of range

220   Unable to open instrument

223   Instrument identification query failed

230   Error writing to instrument

231   Error reading from instrument

232   Instrument not initialized

240   Instrument timed out

4xx   See RS232 Library error codes

(rs232err = 400 - Error)

________________________________________________________________________________

asyc2_voltage

int asyc2_voltage (int function, int range, int RELMode, double referenceVOLT,

int zoom, int hold, int lrhd, int surv);

Purpose

Switches the instrument to voltage measurement.

Parameter List

function

Variable Type       int

Specifies the type of measurement.

Variable Type: integer

Valid Range:   0 -  DC

1 -  AC

2 -  AC+DC

3 -  Peak+

4 -  Peak-

Default Value: 0 -  DC

range

Variable Type       int

Specifies the range of the measurement

(auto ranging is disable with Peak measurement).

Variable Type:  integer

Variable Range:  0 - AUTO

1 - 500 mV

2 - 5 V

3 - 50 V

4 - 500 V

5 - 750/1000 V

Default Value:   0 - AUTO

RELMode

Variable Type       int

Specifies the REL Mode. If it is

- Last Value:

the last value measured becomes the

reference value derived from subsequence

measurement

- Reference:

the reference value is the value

stored in the panel 'Reference'.

Variable Type:  integer

Variable Range:  2 - Reference

1 - Last Value

0 - Off

referenceVOLT

Variable Type       double

Specifies the relative reference value.

Variable Type:   double

Variable Range:  according to Range:

750/1000 V       -1000  to +1000 VOLT

500 V        -500  to  +500 VOLT

50 V         -50  to   +50 VOLT

5 V          -5  to    +5 VOLT

500mV        - 0.5 to    0.5VOLT

AUTO       -1000  to +1000 VOLT

Default Value:   0

zoom

Variable Type       int

Specifies the ZOOM mode to expand the bargraph.

Variable Type:  integer

Variable Range:  0 - Off

1 - On

Default Value:   0 - Off

hold

Variable Type       int

Specifies the MEM mode to fix the display to the

current value.

Variable Type:  integer

Variable Range:  0 - Off

1 - On

Default Value:   0 - Off

lrhd

Variable Type       int

Specifies the LHRD mode to the last reading hold

current value.

Variable Type:  integer

Variable Range:  0 - Off

1 - On

Default Value:   0 - Off

surv

Variable Type       int

Specifies the SURV mode to access the surveillance

mode in which minimum, maximum and sliding average

values of the current measurement are stored.

Variable Type:  integer

Variable Range:  0 - Off

1 - On

Default Value:   0 - Off

Return Value

Displays the results of the instrument driver

function call:

Variable Type: Integer

Error Codes:

0   Success

-1   Config out of range

-2   Range out of range

-3   REL Mode out of range

-4   Reference out of range

-5   Zoom out of range

-6   Memo out of range

-7   Lrhd out of range

-8   Surv out of range

220   Unable to open instrument

223   Instrument identification query failed

230   Error writing to instrument

231   Error reading from instrument

232   Instrument not initialized

240   Instrument timed out

300   Function not available

301   Wrong position of the rotary switch

302   Two arguments are incompatible

303   Measurement is overloaded (OL)

4xx   See RS232 Library error codes

(rs232err = 400 - Error)