Metrix OX8042/62 Oscilloscope (VISA I/O)

________________________________________________________________________________

Introduction:

This instrument driver provides programming support for Metrix OX8042/62 Oscilloscope (VISA I/O).

It contains functions for opening, configuring, taking measurements from, and

closing the instrument.

________________________________________________________________________________

Assumptions:

To successfully use this module, the following conditions must be met:

For RS-232 instrument drivers:

-  the instrument is connected to the RS-232 interface.

-  the COM port, baud rate, parity, and timeout supplied to the

initialize function must match the settings of the instrument.

________________________________________________________________________________

Error and Status Information:

Each function in this instrument driver returns a status code that

either indicates success or describes an error or warning condition.

Your program should examine the status code from each call to an

instrument driver function to determine if an error occurred.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

The description of each instrument driver function lists possible error

codes and their meanings

________________________________________________________________________________

How To Use This Document:

Use this document as a programming reference manual.

It describes each function in the

Metrix OX8042/62 Oscilloscope (VISA I/O)

instrument.  The functions appear in alphabetical order, with a description

of the function and its C syntax, a description of each parameter, and a list

of possible error codes.

________________________________________________________________________________

Function Tree Layout:

Metrix OX8042/62 Oscilloscope (VISA I/O)  Function Name:

Initialize                                  mxOX8042_init

Application Functions

Example                                  mxOX8042_example

Configuration Functions

Read/Write Setup                         mxOX8042_readWriteSetup

Autoscale                                mxOX8042_autoScale

Config Acquisition                       mxOX8042_confAcq

Config Input                             mxOX8042_confInp

Config Horizontal                        mxOX8042_confHoriz

Config Save Trace                        mxOX8042_confSaveTrace

Config Display                           mxOX8042_confDisp

Config Trigger                           mxOX8042_confTrig

Config Delay                             mxOX8042_confDelay

Config Cursors                           mxOX8042_confCursors

Action/Status Functions

Trigger                                  mxOX8042_trigger

Data Functions

Measure Auto                             mxOX8042_measAuto

Measure Cursors                          mxOX8042_measCurs

Read Trace                               mxOX8042_readTrace

Utility Functions

Write To Instrument                      mxOX8042_writeInstrData

Read Instrument Data                     mxOX8042_readInstrData

Reset                                    mxOX8042_reset

Self-Test                                mxOX8042_selfTest

Error-Query                              mxOX8042_errorQuery

Error Message                            mxOX8042_errorMessage

Revision Query                           mxOX8042_revisionQuery

Front Panel Lockout                      mxOX8042_frontPanLock

Close                                       mxOX8042_close

________________________________________________________________________________

Metrix OX8042/62 Oscilloscope (VISA I/O)

This instrument module provides programming support for the Metrix

OX8042/62 Oscilloscope.  The module is divided into the following

functions:

Functions/Classes:

(1) Initialize:

This function initializes the instrument and sets it to a default

configuration.

(2) Application Functions: (Class)

This class contains high-level, test and measurement routines.  These

examples call other instrument driver functions to configure, start, and

read from the instrument.

(3) Configuration Functions: (Class)

This class of functions configures the instrument by setting acquisition

and system configuration parameters.

(4) Action/Status Functions: (Class)

This class of functions begins or terminates an acquisition. It also

provides functions which allow the user to determine the current status

of the instrument.

(5) Data Functions: (Class)

This class of functions transfers data to or from the instrument.

(6) Utility Functions: (Class)

This class of functions provides lower level functions to communicate

with the instrument, and change instrument parameters.

(7) Close:

This function takes the instrument offline.

________________________________________________________________________________

The following functions are in alphabetical order.

________________________________________________________________________________

mxOX8042_autoScale

ViStatus mxOX8042_autoScale (ViSession instrumentHandle);

Purpose

This function initiates the autoscaling operation.

Notes:

(1) In the analog mode, signals are displayed as soon as the autoscale is

complete.

(2) In the digital mode, there are two possibilities depending on the

state of acquisition:

Idle (continuous acquisition is off): only the current settings and the

channel selection are updated on the screen.

Active (continuous acquisition is on): acquisition is automatically

reinitiated after the end of the autoscale.

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

3FFF0005  The specified termination character was read.

3FFF0006  The specified number of bytes was read.

BFFC0803  Error Scanning Response

BFFC0900  Instrument specific error.

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

mxOX8042_close

ViStatus mxOX8042_close (ViSession instrumentHandle);

Purpose

This function performs the following operations:

viClose (instrSession) and viClose (rmSession).

Notes:

(1) The instrument must be reinitialized to use it again.

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

3FFF0005  The specified termination character was read.

3FFF0006  The specified number of bytes was read.

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

mxOX8042_confAcq

ViStatus mxOX8042_confAcq (ViSession instrumentHandle, ViInt32 mode,

ViBoolean glitchDetect, ViInt32 memorySize,

ViBoolean clear);

Purpose

This function configures the acquisition modes of the instrument. The

following settings are accessible:

- acquisition mode, incl. envelope mode

- glitch detect function

- memory size

- clear function

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

mode

Variable Type       ViInt32

This control specifies the acquisition mode of the oscilloscope.

Valid Values:

0 - Refresh  (Default Value)

1 - Roll

2 - Single

3 - Envelope

4 - Analog

Notes:

(1) In the Refresh Mode the instrument works like the traditional

analog oscilloscope.

(2) The Roll Mode is available only if the timebase is greater than

100 ms/div. An error is generated for smaller timebases.

(3) In the Single Mode the instrument acquires a single trace and

stops the acquisition.

(4) The Envelope Mode is available only if the timebase is 100 ms/div

or less. This mode is similar to the refresh mode. The difference is

that the previously acquired signals persist on instrument's display.

Trace data read over the GPIB or serial interface contain the last

acquired signal.

glitchDetect

Variable Type       ViBoolean

This control turns on or off the glitch detect function of the

instrument.

Valid Values:

VI_OFF (0) - Off  (Default Value)

VI_ON  (1) - On

Notes:

(1) The glitch detect feature is available only when in the digital

mode.

memorySize

Variable Type       ViInt32

This control selects the size of acquisition memory.

Valid Values:

0 - 1K  (Default Value)

1 - 8K

2 - 16K

clear

Variable Type       ViBoolean

This control clears the screen of the instrument.

Valid Values:

VI_OFF (0) - Off  (Default Value)

VI_ON  (1) - On

Notes:

(1) The clear function is available only when in the digital mode.

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

3FFF0005  The specified termination character was read.

3FFF0006  The specified number of bytes was read.

BFFC0002  Parameter 2 (Mode) out of range.

BFFC0003  Parameter 3 (Glitch Detect) out of range.

BFFC0004  Parameter 4 (Memory size) out of range.

BFFC0005  Parameter 5 (Clear) out of range.

BFFC0803  Error Scanning Response

BFFC0900  Instrument specific error.

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

mxOX8042_confCursors

ViStatus mxOX8042_confCursors (ViSession instrumentHandle, ViInt32 measure,

ViInt32 cursor1, ViInt32 cursor2,

ViInt32 cursor3);

Purpose

This function forces the instrument to perform selected measurement with

cursors.

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

measure

Variable Type       ViInt32

This control selects the measure with cursors.

Valid Values:

0 - Off  (Default Value)

1 - Volt

2 - Time

3 - Frequency

4 - Phase

cursor1

Variable Type       ViInt32

This control specifies the cursor1 coordinates.

Valid Values:  0 to 250

25 to 225  for Volt measure

Default Value: 0

cursor2

Variable Type       ViInt32

This control specifies the cursor2 coordinates.

Valid Values:  0 to 250

25 to 225  for Volt measure

Default Value: 0

cursor3

Variable Type       ViInt32

This control specifies the cursor3 coordinates.

Valid Values:  0 to 250

Default Value: 0

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

3FFF0005  The specified termination character was read.

3FFF0006  The specified number of bytes was read.

BFFC0002  Parameter 2 (Measure) out of range.

BFFC0003  Parameter 3 (Cursor1) out of range.

BFFC0004  Parameter 4 (Cursor2) out of range.

BFFC0005  Parameter 5 (Cursor3) out of range.

BFFC0803  Error Scanning Response

BFFC0900  Instrument specific error.

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

mxOX8042_confDelay

ViStatus mxOX8042_confDelay (ViSession instrumentHandle, ViInt32 delay);

Purpose

This function configures the timebase B system. An error is generated if

the conditions mentioned above are not met.

The following setting is configured with this function:

- delay mode

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

delay

Variable Type       ViInt32

This control selects the delay mode.

Valid Values:

0 - Off  (Default Value)

1 - Search

2 - Del

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

3FFF0005  The specified termination character was read.

3FFF0006  The specified number of bytes was read.

BFFC0002  Parameter 2 (Delay) out of range.

BFFC0803  Error Scanning Response

BFFC0900  Instrument specific error.

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

mxOX8042_confDisp

ViStatus mxOX8042_confDisp (ViSession instrumentHandle, ViInt32 displayMode,

ViInt32 windowOrigin);

Purpose

This function performs the instrument's display configuration. The

following settings are accessible via this function:

- display mode

- screen window origin

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

displayMode

Variable Type       ViInt32

This control selects the trace's display mode of the instrument.

Valid Values:

0 - CH1

1 - ALT

2 - CHOP  (Default Value)

3 - CH2

4 - XY

5 - ADD

6 - Multiply

windowOrigin

Variable Type       ViInt32

This control specifies the screen window origin in the scope's

digital mode. It has no effect in the analog mode.

Valid Range: 0 to 15800

Default Value: 0

Notes:

(1) Zero is the left end of the signal sweep.

800 is the right end for 1k.

7800 is the right end for 8k.

15800 is the right end for 16k.

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

3FFF0005  The specified termination character was read.

3FFF0006  The specified number of bytes was read.

BFFC0002  Parameter 2 (Display Mode) out of range.

BFFC0003  Parameter 3 (Window Origin) out of range.

BFFC0803  Error Scanning Response

BFFC0900  Instrument specific error.

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

mxOX8042_confHoriz

ViStatus mxOX8042_confHoriz (ViSession instrumentHandle, ViInt32 timebase,

ViInt32 expansion);

Purpose

This function configures the horizontal settings of the instrument.

The following settings are available:

- range (the actual timebase)

- horizontal expansion

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

timebase

Variable Type       ViInt32

This control sets the timebase of the oscilloscope.

Valid Values:

0 - 5 ns/div

1 - 10 ns/div

2 - 20 ns/div

3 - 50 ns/div

4 - 0.1 us/div

5 - 0.2 us/div

6 - 0.5 us/div

7 - 1 us/div

8 - 2 us/div

9 - 5 us/div  (Default Value)

10 - 10 us/div

11 - 20 us/div

12 - 50 us/div

13 - 100 us/div

14 - 200 us/div

15 - 500 us/div

16 - 1 ms/div

17 - 2 ms/div

18 - 5 ms/div

19 - 10 ms/div

20 - 20 ms/div

21 - 50 ms/div

22 - 100 ms/div

23 - 200 ms/div

expansion

Variable Type       ViInt32

This control sets the horizontal expansion factor in digital as well

as in the analog mode.

Valid Values:  See the table below.

Default Value: 9 - No Change

Setting      Factor             Points Displayed

=============================================================

0            x 0.05 or x 0.04   20000 or 25000

1            x 0.1              10000

2            x 0.2  or x 0.25    5000 or 4000

3            x 0.5  or x 0.4     2000 or 2500

4            x 1.0               1000

5            x 2.0  or x 2.5      500 or 400

6            x 5.0  or x 4.0      250 or 200

7            x 1  (analog mode)

8            x 10 (analog mode)

9            no change

Notes:

(1) Use setting No Change (9) if you want to change the timebase on

the fly (when acquisition is running). Failing to do so will generate

an error.

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

3FFF0005  The specified termination character was read.

3FFF0006  The specified number of bytes was read.

BFFC0002  Parameter 2 (Timebase) out of range.

BFFC0003  Parameter 3 (Expansion) out of range.

BFFC0803  Error Scanning Response

BFFC0900  Instrument specific error.

BFFC0903  Error setting expansion when acquisition is running.

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

mxOX8042_confInp

ViStatus mxOX8042_confInp (ViSession instrumentHandle, ViInt32 channel,

ViInt32 voltageRange, ViInt32 coupling,

ViBoolean mode, ViInt32 probe);

Purpose

This function configures the selected input channel of the instrument.

The following settings are accessible:

- channel

- coupling

- range

- mode normal / differential

- probe factor

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

channel

Variable Type       ViInt32

This control selects the channel to configure.

Valid Values:

1 - CH1  (Defaul Value)

2 - CH2

voltageRange

Variable Type       ViInt32

This control sets the input range of the selected channel in units of

volts per division.

Valid Values:

0 -   10 mV/div

1 -   20 mV/div

2 -   50 mV/div

3 -  100 mV/div

4 -  200 mV/div

5 -  500 mV/div

6 -    1  V/div

7 -    2  V/div

8 -    5  V/div

9 -   10  V/div     (Default Value)

10 -  20  V/div

11 -  50  V/div

12 - 100  V/div

13 - 200  V/div

coupling

Variable Type       ViInt32

This control selects the input coupling of the selected channel.

Valid Values:

0 - AC  (Default Value)

1 - DC

2 - GND

mode

Variable Type       ViBoolean

This control specifies whether to switch the channel in differential

mode.

Valid Value:

VI_OFF (0) - Normal  (Default Value)

VI_ON  (1) - Differential

probe

Variable Type       ViInt32

This control selects the probe factor of the selected channel.

Valid Values:

0 - X 1  (Default Value)

1 - X 10

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

3FFF0005  The specified termination character was read.

3FFF0006  The specified number of bytes was read.

BFFC0002  Parameter 2 (Channel) out of range.

BFFC0003  Parameter 3 (Voltage Range) out of range.

BFFC0004  Parameter 4 (Coupling) out of range.

BFFC0005  Parameter 5 (Mode) out of range.

BFFC0006  Parameter 6 (Probe) out of range.

BFFC0803  Error Scanning Response

BFFC0900  Instrument specific error.

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

mxOX8042_confSaveTrace

ViStatus mxOX8042_confSaveTrace (ViSession instrumentHandle, ViInt32 trace,

ViBoolean save);

Purpose

This function controls the access to the trace memories (CH1, CH2). It

has an effect only when in the digital mode.

Notes:

(1) This function will perform properly only if the acquisition is not

running. If this function is called when acquisition is running, an error

is generated. Stop the acquisition using function Trigger.

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

trace

Variable Type       ViInt32

This control selects the memory to be write-protected or not.

Valid Values:

1 - CH1  (Default Value)

2 - CH2

save

Variable Type       ViBoolean

This control specifies whether to save the trace to the memory

(turning memory on) or to turn the memory off.

Valid Values:

VI_OFF (0) - Off  (Default Value)

VI_ON  (1) - On

Notes:

(1) To save the trace to the memory the acquisition must be in the

idle mode (continuous acquisition is off). Attempt to save trace in

the continuous acquisition mode generates an error.

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

3FFF0005  The specified termination character was read.

3FFF0006  The specified number of bytes was read.

BFFC0002  Parameter 2 (Trace) out of range.

BFFC0003  Parameter 3 (Save) out of range.

BFFC0803  Error Scanning Response

BFFC0900  Instrument specific error.

BFFC0905  Attempt to save trace when the acquisition is running.

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

mxOX8042_confTrig

ViStatus mxOX8042_confTrig (ViSession instrumentHandle, ViInt32 source,

ViInt32 coupling, ViInt32 pretriggerDelay,

ViBoolean peaktoPeak, ViBoolean slope,

ViBoolean mode);

Purpose

This function configures the triggering system of the instrument. The

following settings can be configured with this function:

- trigger source

- coupling

- peak to peak

- trigger slope

- pretrigger or delay

- trigger mode

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

source

Variable Type       ViInt32

This control selects the trigger source. See the notes below to get

more information.

Valid Values:

0 - Channel 1  (Default Value)

1 - Channel 2

2 - Display Dependent

3 - Line

4 - External

Notes:

(0) The syncronization source is selected in the following manner:

Channel 1:  Synchronized on channel 1.

Channel 2:  Synchronized on channel 2.

Display Dependent:  The trigger source is defined by the display

mode, see the table below for more information:

display mode            trigger source

======================================================

CH1                         CH1

ALT               channel 1 synchronized with CH1

channel 2 synchronized with CH2

CHOP                        CH1

ADD (CH1+CH2)               CH1

CH2                         CH2

-CH2                         CH2

Line:      Synchronized on the mains power sypply.

External:  Synchronized on external source via the EXT connector

on the front panel of the instrument.

coupling

Variable Type       ViInt32

This control specifies the trigger signal coupling. See the note

below for more information.

Valid Values:

0 - DC  (Default Value)

1 - AC

2 - LF

3 - HF

4 - TVH

5 - TVV

Notes:

(0)  DC coupling (0 to 40 MHz)

(1)  AC coupling (10 Hz to 40 MHz)

(2)  Rejects frequencies below 10 kHz from the source signal

(facilitates observation of signals with DC component).

(3)  Rejects frequencies above 10 kHz from the source signal

(facilitates observation of signals with high frequency noise).

pretriggerDelay

Variable Type       ViInt32

This control specifies either the pretrigger or the delay state of

the triggering system of the instrument. See the notes below for more

information.

Valid Values:

-1 - manual delay

0 -     0 point   (Default Value)

1 -   250 points  (1K only)

2 -   500 points  (1K only)

3 -   750 points  (1K only)

4 -  1000 points

5 -  2000 points

6 -  3000 points

7 -  4000 points

8 -  5000 points

9 -  6000 points

10 -  7000 points

11 -  8000 points

12 -  9000 points  (16K only)

13 - 10000 points  (16K only)

14 - 11000 points  (16K only)

15 - 12000 points  (16K only)

16 - 13000 points  (16K only)

17 - 14000 points  (16K only)

18 - 15000 points  (16K only)

19 - 16000 points  (16K only)

Notes:

(1) Settings 0 to 19 specify the pretrigger value in points (has no

effect in the analog mode).

(2) Setting 0 turns off the pretrigger AND delay.

(3) Setting -1 turns the delay status on, i.e. the delay value can be

changed using DELAY knob on the front panel of the instrument.

peaktoPeak

Variable Type       ViBoolean

This control specifies the Peak to peak on the trigger source.

Valid Values:

VI_OFF (0) - Off  (Default Value)

VI_ON  (1) - On

slope

Variable Type       ViBoolean

This control specifies the slope of the trigger source.

Valid Values:

VI_OFF (0) - Negative

VI_ON  (1) - Positive  (Default Value)

mode

Variable Type       ViBoolean

This control selects the auto mode of the instrument.

Valid Values:

VI_OFF - Normal

VI_ON  - Auto  (Default Value)

Notes:

(0)  The trigger level is set manually via the knob labeled LEVEL on

the front panel of the instrument. The range is approx. -20div to 20

div.

(1)  In the Auto mode the trigger events are generated regardless of

the triggering conditions, the measured signal is not synchronized

with the timebase.

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

3FFF0005  The specified termination character was read.

3FFF0006  The specified number of bytes was read.

BFFC0002  Parameter 2 (Source) out of range.

BFFC0003  Parameter 3 (Coupling) out of range.

BFFC0004  Parameter 4 (Pretrigger/Delay) out of range.

BFFC0005  Parameter 5 (Peak to peak) out of range.

BFFC0006  Parameter 6 (Slope) out of range.

BFFC0007  Parameter 7 (Mode) out of range.

BFFC0803  Error Scanning Response

BFFC0900  Instrument specific error.

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

mxOX8042_errorMessage

ViStatus mxOX8042_errorMessage (ViSession instrumentHandle, ViStatus statusCode,

ViChar message[]);

Purpose

This function takes the Status Code returned by the instrument driver

functions, interprets it and returns it as a user readable string.

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  VI_NULL

statusCode

Variable Type       ViStatus

This control accepts the Status Code returned from the instrument

driver functions.

Default Value:

0 - VI_SUCCESS

message

Variable Type       ViChar[]

This control returns the interpreted Status Code as a user readable

message string.

Notes:

(1) The array must contain at least 256 elements ViChar[256].

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

3FFF0005  The specified termination character was read.

3FFF0006  The specified number of bytes was read.

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

mxOX8042_errorQuery

ViStatus mxOX8042_errorQuery (ViSession instrumentHandle, ViInt32 *errorCode,

ViChar errorMessage[]);

Purpose

This function reads an error code from the instrument's error queue.

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

errorCode

Variable Type       ViInt32 (passed by reference)

This control returns the error code read from the instrument's error

queue.

errorMessage

Variable Type       ViChar[]

This control returns the error message string read from the

instrument's error message queue.

Notes:

(1) The array must contain at least 256 elements ViChar[256].

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

3FFF0005  The specified termination character was read.

3FFF0006  The specified number of bytes was read.

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

mxOX8042_example

ViStatus mxOX8042_example (ViSession instrumentHandle, ViInt32 timebase,

ViInt32 voltageRange, ViInt32 coupling,

ViReal64 data[], ViReal64 *deltaT,

ViInt32 *numberofSamples);

Purpose

This function is an example of this driver's functions use. It works as a

simple single channel digitizer.

The instrument is set to the single shot digital mode in dual

channel configuration. User can change the timebase (Timebase), the input

voltage range (Voltage Range) and the input coupling (Coupling).

A trigger is then sent to the instrument. The function Trigger waits

until the acquisition is completed. Channel S1 is read out from the

instrument and displayed in the graph after the acquisition completes.

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

timebase

Variable Type       ViInt32

This control sets the timebase of the oscilloscope.

Valid Values:

0 - 5 ns/div

1 - 10 ns/div

2 - 20 ns/div

3 - 50 ns/div

4 - 0.1 us/div

5 - 0.2 us/div

6 - 0.5 us/div

7 - 1 us/div

8 - 2 us/div

9 - 5 us/div

10 - 10 us/div

11 - 20 us/div

12 - 50 us/div  (Default Value)

13 - 100 us/div

14 - 200 us/div

15 - 500 us/div

16 - 1 ms/div

17 - 2 ms/div

18 - 5 ms/div

19 - 10 ms/div

20 - 20 ms/div

21 - 50 ms/div

22 - 100 ms/div

23 - 200 ms/div

voltageRange

Variable Type       ViInt32

This control sets the input range of the selected channel in units of

volts per division.

Valid Values:

0 - 10 mV/div

1 - 20 mV/div

2 - 50 mV/div

3 - 100 mV/div

4 - 200 mV/div

5 - 500 mV/div

6 - 1 V/div

7 - 2 V/div

8 - 5 V/div

9 - 10 V/div     (Default Value)

10 - 20 V/div

11 - 50 V/div

12 - 100 V/div

13 - 200 V/div

coupling

Variable Type       ViInt32

This control selects the input coupling of the selected channel.

Valid Values:

0 - AC  (Default Value)

1 - DC

2 - GND

data

Variable Type       ViReal64[]

Returns an array of the trace S1 data in units of volts.

Notes:

(1) The array must contain at least 1000 elements ViReal64[1000].

deltaT

Variable Type       ViReal64 (passed by reference)

This control returns the time difference between two samples of the

trace data in units of seconds.

numberofSamples

Variable Type       ViInt32 (passed by reference)

This control returns number of samples acquired by the instrument. If

the function executes successfully, value returned by this control is

equal to a number of valid samples contained within control data.

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

BFFC0002  Parameter 2 (Timebase) out of range.

BFFC0003  Parameter 3 (Voltage Range) out of range.

BFFC0004  Parameter 4 (Coupling) out of range.

3FFF0005  The specified termination character was read.

3FFF0006  The specified number of bytes was read.

BFFC0803  Error Scanning Response

BFFC0900  Instrument specific error.

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

mxOX8042_frontPanLock

ViStatus mxOX8042_frontPanLock (ViSession instrumentHandle, ViBoolean mode);

Purpose

This function controls the instrument's front panel lockout state.

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

mode

Variable Type       ViBoolean

Controls the front panel lockout.

Valid Values:

VI_OFF (0) - Unlock (Default Value)

VI_ON  (1) - Lock

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

3FFF0005  The specified termination character was read.

3FFF0006  The specified number of bytes was read.

BFFC0002  Parameter 2 (Mode) out of range.

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

mxOX8042_init

ViStatus mxOX8042_init (ViRsrc resourceName, ViBoolean IDQuery,

ViBoolean resetDevice, ViSession *instrumentHandle);

Purpose

This function performs the following initialization actions:

- Opens a session to the Default Resource Manager resource and a session

to the specified device using the interface and address specified in the

Resource Name control.

- Performs an identification query on the Instrument.

- Resets the instrument to a known state.

- Sends initialization commands to the instrument that set any necessary

programmatic variables such as Headers Off, Short Command form, and Data

Transfer Binary to the state necessary for the operation of the

instrument driver.

- Returns an Instrument Handle which is used to differentiate between

different sessions of this instrument driver.

- Each time this function is invoked a Unique Session is opened.  It is

possible to have more than one session open for the same resource.

Parameter List

resourceName

Variable Type       ViRsrc

This control specifies the interface and address of the device that

is to be initialized (Instrument Descriptor). The exact grammar to be

used in this control is shown in the note below.

Default Value:  "ASRL1"

Notes:

(1) Based on the Instrument Descriptor, this operation establishes a

communication session with a device.  The grammar for the Instrument

Descriptor is shown below.  Optional parameters are shown in square

brackets ([]).

Interface   Grammar

------------------------------------------------------

Serial      ASRLserialport[::INSTR]

The ASRL keyword is used with serial instruments.

IDQuery

Variable Type       ViBoolean

This control specifies if an ID Query is sent to the instrument

during the initialization procedure.

Valid Range:

VI_OFF (0) - Skip Query

VI_ON  (1) - Do Query (Default Value)

Notes:

(1) Under normal circumstances the ID Query ensures that the

instrument initialized is the type supported by this driver. However

circumstances may arise where it is undesirable to send an ID Query

to the instrument.  In those cases; set this control to "Skip Query"

and this function will initialize the selected interface, without

doing an ID Query.

resetDevice

Variable Type       ViBoolean

This control specifies if the instrument is to be reset to its

power-on settings during the initialization procedure.

Valid Range:

VI_OFF (0) - Don't Reset

VI_ON  (1) - Reset Device (Default Value)

Notes:

(1) If you do not want the instrument reset. Set this control to

"Don't Reset" while initializing the instrument.

instrumentHandle

Variable Type       ViSession (passed by reference)

This control returns an Instrument Handle that is used in all

subsequent function calls to differentiate between different sessions

of this instrument driver.

Notes:

(1) Each time this function is invoked a Unique Session is opened.

It is possible to have more than one session open for the same

resource.

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

3FFF0005  The specified termination character was read.

3FFF0006  The specified number of bytes was read.

BFFC0002  Parameter 2 (ID Query) out of range.

BFFC0003  Parameter 3 (Reset Device) out of range.

BFFC0011  Instrument returned invalid response to ID Query

BFFC0803  Error Scanning Response

BFFC0900  Instrument specific error.

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

mxOX8042_measAuto

ViStatus mxOX8042_measAuto (ViSession instrumentHandle, ViInt32 function,

ViInt32 source, ViReal64 *output);

Purpose

This function forces the instrument to perform selected measurement and

returns the result.

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

function

Variable Type       ViInt32

This control specifies the measurement function.

Valid Values:

0 - Off    (Default Value)

1 - Root Mean Square Voltage [V]

2 - Average Voltage [V]

3 - Frequency [Hz]

4 - Period [sec]

5 - Phase [deg]

6 - Amplitude [V]

7 - Low Voltage [V]

8 - High Voltage [V]

9 - Rise Time [sec]

10 - Fall Time [sec]

11 - Negative Pulse Width [sec]

12 - Positive Pulse Width [sec]

13 - Positive Duty Cycle [%]

14 - Negative Duty Cycle [%]

15 - Minimum Voltage [V]

16 - Maximum Voltage [V]

17 - Peak To Peak Voltage [V]

Notes:

(1) When measuring phase, make sure the instrument is in dual mode

(phase shift between two channels is measured). The phase value is

output in the form:

phase = source CH phase - complementary CH phase.

source

Variable Type       ViInt32

This control specifies the source channel for the measurement.

Valid Values:

1 - CH1  (Default Value)

2 - CH2

output

Variable Type       ViReal64 (passed by reference)

This control outputs the result of the measurement. All measured

quantities are output in their basic units.

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

3FFF0005  The specified termination character was read.

3FFF0006  The specified number of bytes was read.

BFFC0002  Parameter 2 (Function) out of range.

BFFC0003  Parameter 3 (Source) out of range.

BFFC0803  Error Scanning Response

BFFC0900  Instrument specific error.

BFFC0902  Measurement failed (See User's manual).

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

mxOX8042_measCurs

ViStatus mxOX8042_measCurs (ViSession instrumentHandle, ViInt32 source,

ViReal64 *output);

Purpose

This function returns the result of measure with cursors.

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

source

Variable Type       ViInt32

This control specifies the source channel for the measurement.

Valid Values:

1 - CH1  (Default Value)

2 - CH2

output

Variable Type       ViReal64 (passed by reference)

This control outputs the result of the measurement. All measured

quantities are output in their basic units.

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

3FFF0005  The specified termination character was read.

3FFF0006  The specified number of bytes was read.

BFFC0002  Parameter 2 (Source) out of range.

BFFC0803  Error Scanning Response

BFFC0900  Instrument specific error.

BFFC0902  Measurement failed (See User's manual).

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

mxOX8042_readInstrData

ViStatus mxOX8042_readInstrData (ViSession instrumentHandle,

ViInt16 numberBytesToRead, ViChar readBuffer[],

ViInt32 *numBytesRead);

Purpose

This function reads data from the instrument's output buffer and returns

it to the specified variable in memory.

Notes:

(1) Because the instrument may return both numeric and text data in

response to queries this function returns the data in string format.

(2) If valid data is not available at the instrument's output buffer when

this function is called the instrument will hang up and the function will

not return until it times out.  If the time-out is disabled this function

will hang indefinitely and it may be necessary to reboot the computer to

break out.

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

numberBytesToRead

Variable Type       ViInt16

The number of bytes specified by this control should be greater than

or equal to the number of bytes which are to be read from the

instrument. If the actual number of bytes to be read is greater than

the number this control specifies then multiple reads will be

required to empty the instrument's output buffer.

If the instrument's output buffer is not emptied the instrument may

return invalid data when future reads are performed.

Default Value:  50 (See NOTE 2)

Notes:

(1) If NO DATA is available at the instrument's output buffer when

this function is called the instrument will hang up and the function

will not return until it times out.  If the time-out is disabled,

this function will hang indefinitely and it may be necessary to

reboot the computer.

(2) If the number of bytes expected is greater than 50 the value of

this control may be increased. If the actual number of bytes read is

larger than the declared size of the read buffer a run-time error

will be generated.

readBuffer

Variable Type       ViChar[]

The incoming data from the instrument is placed into this variable.

Notes:

(1) Because the instrument may return both numeric and text data in

response to queries this function returns the data in string format.

(2) This function does not overwrite any old data left in the string

variable from the last time the function was called. String data in

LabWindows/CVI is terminated with an ASCII null(0x0) and string

manipulation functions will only recognize data before the ASCII

null.

(3) The declared size of the string variable must be greater than the

actual number of bytes read from the instrument if it is not a

run-time error will be generated.

numBytesRead

Variable Type       ViInt32 (passed by reference)

This variable contains the actual number of bytes read from the

instrument. This is the value which is returned by the read function.

Notes:

(1) If the actual number of bytes read is less than the number of

bytes specified in the Number Bytes To Read control then the output

buffer has probably been emptied. If the read function fails and the

number of bytes read is 0, the most probable cause for the failure is

there was no data available at the instrument's output buffer.

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

3FFF0005  The specified termination character was read.

3FFF0006  The specified number of bytes was read.

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

mxOX8042_readTrace

ViStatus mxOX8042_readTrace (ViSession instrumentHandle, ViInt32 trace,

ViReal64 traceData[], ViInt32 *numberOfSamples,

ViReal64 *t0, ViReal64 *dt);

Purpose

This function reads the trace data from the instrument.

Notes:

(1) If the acquisition of the oscilloscope runs, the oscilloscope doesn't

respond to Read Trace query. To avoid this, stop the acquisition

(function Trigger) before reading trace from the oscilloscope. Failing to

stop acquisition before reading trace will result in an error.

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

trace

Variable Type       ViInt32

This control specifies the trace to be read out.

Valid Values:

0 - CH1 saved (Default Value)

1 - CH2

2 - S1: signal 1 displayed

3 - S2: signal 2 displayed

4 - CH1L:CH1 with Envelope Mode (Low Peak)

5 - CH1H:CH1 with Envelope Mode (High Peak)

6 - CH2L:CH2 with Envelope Mode (Low Peak)

7 - CH2H:CH2 with Envelope Mode (High Peak)

traceData

Variable Type       ViReal64[]

This control returns an array of values representing the trace data.

The values are expressed in units of volts.

Notes:

(1) When reading traces S1, S2, the array must contain at least 1000

elements ViReal64[1000].

(2) When reading traces other than S1, S2, the array must contain at

least 1000, 8000 or 16000 elements ViReal64[1000], ViReal64[8000] or

ViReal64[16000].

numberOfSamples

Variable Type       ViInt32 (passed by reference)

This control returns a number of samples of the trace data. If this

function executes properly, it returns 1000, 8000 or 16000.

t0

Variable Type       ViReal64 (passed by reference)

This control returns the time origin of the trace data. It is always

zero.

dt

Variable Type       ViReal64 (passed by reference)

This control returns the time difference between two samples of the

trace data in units of seconds.

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

3FFF0005  The specified termination character was read.

3FFF0006  The specified number of bytes was read.

BFFC0002  Parameter 2 (Trace) out of range.

BFFC0803  Error Scanning Response

BFFC0900  Instrument specific error.

BFFC0901  Selected trace is not available.

BFFC0904  Attempt to read trace when acquisition is running.

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

mxOX8042_readWriteSetup

ViStatus mxOX8042_readWriteSetup (ViSession instrumentHandle, ViBoolean mode,

ViChar instrumentSetup[]);

Purpose

This function reads/writes the instrument setup data from/to the

instrument. The setup data being read from the instrument always contains

current configuration.

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

mode

Variable Type       ViBoolean

This control specifies whether to read or write the instrument setup

string.

Valid Values:

VI_OFF (0) - Write

VI_ON  (1) - Read  (Default Value)

instrumentSetup

Variable Type       ViChar[]

This control contains the binary setup data. When reading the setup

data, the setup data is output via this control. To write the setup

data, use setup strings read from the instrument before.

The setup string is 72 characters long. If the setup data contained

in the string are invalid on writes an error is generated.

Default Value: ""

Notes:

(1) The array must contain at least 72 elements ViChar[80].

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

3FFF0005  The specified termination character was read.

3FFF0006  The specified number of bytes was read.

BFFC0002  Parameter 2 (Mode) out of range.

BFFC0803  Error Scanning Response

BFFC0900  Instrument specific error.

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

mxOX8042_reset

ViStatus mxOX8042_reset (ViSession instrumentHandle);

Purpose

This function resets the instrument to a power-on default state.  Consult

the user's manual for the default parameter settings to which this

command resets the instrument.

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

3FFF0005  The specified termination character was read.

3FFF0006  The specified number of bytes was read.

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

mxOX8042_revisionQuery

ViStatus mxOX8042_revisionQuery (ViSession instrumentHandle,

ViChar instrumentDriverRevision[],

ViChar firmwareRevision[]);

Purpose

This function returns the revision numbers of the instrument driver and

instrument firmware, and tells the user with which  instrument firmware

this revision of the driver is compatible.

This instrument driver's Revision Number is "Rev 1.0, 04/1999, CVI 5.0.1"

and is compatible with Instrument Firmware Revision "1.00".

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

instrumentDriverRevision

Variable Type       ViChar[]

This control returns the Instrument Driver Software Revision.

Notes:

(1) The array must contain at least 256 elements ViChar[256].

firmwareRevision

Variable Type       ViChar[]

This control returns the Instrument Firmware Revision.

Notes:

(1) The array must contain at least 256 elements ViChar[256].

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

3FFF0005  The specified termination character was read.

3FFF0006  The specified number of bytes was read.

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

mxOX8042_selfTest

ViStatus mxOX8042_selfTest (ViSession instrumentHandle, ViInt16 *selfTestResult,

ViChar selfTestMessage[]);

Purpose

This function runs the instrument's self test routine and returns the

test result(s).

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

selfTestResult

Variable Type       ViInt16 (passed by reference)

This control contains the value returned from the instrument self

test. Zero means success. For any other code, see the device's

operator's manual.

selfTestMessage

Variable Type       ViChar[]

This control contains the string returned from the self test. See the

device's operation manual for an explanation of the string's

contents.

Notes:

(1) The array must contain at least 256 elements ViChar[256].

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

3FFF0005  The specified termination character was read.

3FFF0006  The specified number of bytes was read.

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

mxOX8042_trigger

ViStatus mxOX8042_trigger (ViSession instrumentHandle, ViInt32 mode,

ViBoolean wait);

Purpose

This function controls the triggering system of the instrument.

Triggering the oscilloscope runs the acquisition.

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

mode

Variable Type       ViInt32

This control controls the acquisition system of the instrument.

Valid Values:

0 - Continuous  (Default Value)

1 - Single Shot

2 - Abort

Notes:

(1) In the analog mode the interface controlled triggering has no

effect.

(2) In the continuous mode the instrument uses triggers generated by

the trigger source as specified in function Config Trigger. The

continuous acquisition can be aborted by setting abort to this

control. To resume the aborted acquisition use either continuous or

single shot.

(3) When in the roll acquisition mode the option continuous changes

the acquisition mode to refresh. The single shot trigger enables to

start the roll mode acquisition, but it does not start until the

triggering conditions as specified in the function Config Trigger are

met. The acquisition can be aborted by setting abort to this control.

To resume the aborted acquisition use the option single shot.

(4) When in the single acquisition mode the option continuous changes

the acquisition mode to refresh. The single shot trigger enables to

start the roll mode acquisition, but it does not start until the

triggering conditions as specified in the function Config Trigger are

met. The acquisition can be aborted by setting abort to this control.

To resume the aborted acquisition use the option single shot.

(5) If you want to use the instrument as a computer triggered

digitizer, set the control mode in the function Config Trigger to

auto and use the single acquisition mode. As a trigger use function

Trigger with mode set to single shot.

wait

Variable Type       ViBoolean

This control selects whether to wait until a single shot operation

(in single or roll mode) is completed.

Valid Values:

VI_OFF (0) - No  (Default Value)

VI_ON  (1) - Yes

Notes:

(1) This control is valid only if the single shot trigger mode is

selected.

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

3FFF0005  The specified termination character was read.

3FFF0006  The specified number of bytes was read.

BFFC0002  Parameter 2 (Mode) out of range.

BFFC0003  Parameter 3 (Wait?) out of range.

BFFC0803  Error Scanning Response

BFFC0900  Instrument specific error.

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

mxOX8042_writeInstrData

ViStatus mxOX8042_writeInstrData (ViSession instrumentHandle,

ViString writeBuffer);

Purpose

This function writes commands and queries to the instrument to modify

parameters and query device settings.

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

writeBuffer

Variable Type       ViString

The user can use this control to send common commands and queries to

the instrument. This control can also be used to write any valid

command to the instrument.

Default Value:  "*RST\n"

Notes:

(1) The command or query to be sent to the instrument may be a

literal enclosed in double quotes i.e. "*RST" or may be contained in

a variable of type string.

(2) Command strings must be terminated with Linefeed character (its

ASCII code is 10, C language notation is "\n").

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

3FFF0005  The specified termination character was read.

3FFF0006  The specified number of bytes was read.

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.